SAO Voja4 Adapter Notes V1.0

Kit Components

| REF. DES. | DESCRIPTION | COMPONENT |
| --- | --- | --- |
| C1, C3 | Capacitor, 1uF |  |
| C2 | Capacitor, 0.01uF |  |
| J1 | BYOB Battery 2.5-5.5 V |  |
| J2 | 0.1 inch female edge connect, 12-position |  |
| Q1-4 | Transistor, NPN, MMBT4401LT1G |  |
| R1, R4 | Resistor, 0805, 470 Ohm |  |
| R2,R3,R5,R6 | Resistor, 0805, 3K3 Ohm |  |
| U1 | LDO Regulator 3V3, SSP9193-33MSR |  |
| X1 | SAO Socket, Sullins SFH11-NBPC-D03-ST-BK, female, keyed |  |
| X2 | Any 0.1 inch pitch female header, 9-pin |  |
| X3 | Any 0.1 inch pitch female header, 11-pin |  |
| X4 | QWIIC Socket, 4-pos, male, keyed |  |

Configuration Table

| FEATURE | JP1 | JP2,4 | JP3,5 | JP6,7 | Q4 | JP10-13 | JP20-21 | JP22-23 | JP30 | Blue Wire Q4 Base to JP30 pad 3 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| I2C | ~ | Close | Close | ~ | In | Open | ~ | Open | Open | ~ |
| SPI, Single | ~ | ~ | Open | Open | ~ | Close | ~ | Close | Close 1-2 | ~ |
| SPI, Double * | ~ | Close | Open | Open | Out | Close | ~ | Close | Close 1-2 | Close |
| Alternate Power ** | Close | ~ | ~ | ~ | ~ | ~ | ~ | ~ | ~ | ~ |

* Dual SPI mode uses a single OUT1 line to toggle both CS1 and CS2 lines, with opposite logic.

| OUT1 | CS1 | CS2 |
| --- | --- | --- |
| LOW | LOW | LOW |
| HIGH | HIGH | HIGH |

** If using Alternate Power, limit alternate power voltage between 2.5 and 5.5V, and remove the batteries from the Voja4 Badge. This feature may be used with any of the other configuration features.